Settings and Calibration

UPA

1. Calibration and setting

1.1 Turn on the scale.

1.2 Press the password

7	5	9

continuously before the

scale begin to do self-check.

1.3 Utill the weight window shows ‘CAL--1’,then,

Press the key

M+	to change the division.

Press the key

MC	to move decimal point of capacity and press the

numerical key to input the capacity you need.

1.4 Press the key	CG

to confirm and enter the calibration.

1.5 While the weight window shows ‘CAL--2’, please place the weight on the

pan and input the value of weight by numerical key.

1.6 Press the key	CG

,and the weight window will show the value of the

weight, then finish the calibration.

2. Settings

Turn on the scale.

Press and hole the key  +   , till it sound bi-bi..

2.1 press   +   +   .   to choose decimal point

2.2 press  +   and   ZERO to choose zero range