UWA-S CALIBRATION

Press the key  to start the scale. While it display 11111 to

99999, promptly press the key  ,  ,  one by

one.When it shows [F4-CAL],it means enter into the calibration.

- Fixed capacity calibration（5kg）

1.1  Press the key  to show [5.000].

1.2  Press the key  to show [put 1  x].

1.3  Place 5 kg weight on the pan.

1.4  Press the key  to show [F4-CAL].

Then restart the scale to finish calibration.

- Set calibration capacity according to the weight

2.1  Press the key  to show [5.000].

2.2  Press the key  to show [CAL  1].

2.3  Press the key  to show [000.000].

Press the key  to move cursor and press the key  to set the value of calibration weight.

  - Press the key  to show [put 1  x].

Now place the calibration weight on the pan.

  - Press the key  to show [F4-CAL].

Then restart the scale to finish calibration.