Setting and Calibration for

Price Computing Scale UPA-M

1.  Before the calibration,please make sure the scale is under the mode of

‘dt on’.

While you start the scale ,press both the key	+

select mode of ‘dt on’ and ‘dt oF’.

and the key	to

Only under the mode of the ‘dt on’, the scale can enter the mode of setting and calibration.

If you turn to the mode of ‘dt oF’,the mode of setting and calibration will be locked.

2.Press and hold the key

+	while it it starting the power

2.1 Press the key

·	to select digits of unit price.

2.2 Press the key	to select the way to input unit price.

2.3 Press the key

→0←

to select range of zero point.

3. Calibration and setting

3.1 Press the key

ON

OFF

to start the scale.

3.2 Press the password

7	5	9

continuously before the

scale begin to do self-check.

3.3 Utill the weight window shows ‘CAL--1’,then,

Press the key

M+	to move the cursor.

Press the key

MC	to change the divison and press the numerical key to

input the capacity.

Please input the capacity: CAL-1   20.000

Division:   5

3.4 Press the key	CG

calibration.

to confirm and enter the next setting of weight

3.5 While the weight window shows ‘CAL--2’, please place the weight on the

pan and input the value of weight by numerical key.

Please put a 20kg weight on the pan,input the figure: 12.245; this way can set 20viss;

3.6 Press the key	CG

,and the weight window will show the value of the

weight,then finish the calibration.