Setting and Calibration for

Price Computing Scale UPA-T

- Open the cover, to confirm that it is 110V or 220V

- Plug in power, to see whether the charge is valid.

- Open calibration switch, Charging lamp Would run up.

The window will display “CAL—1” when you turn on the scale.

Press the key”V1”to change the division.

Press the key”V2” to select decimal point position.

Press the key”V3”to choose the accuracy of single precision, double precision, third degree.

Then press the key”$” to confirm it.

While the unit price window shows ‘CAL--2’, please place the weight on the

pan and input the value of weight by numerical key. press the key”$” to save after display digital stability, will back to weighing condition.

- Close calibration switch

  - Internal parameter setting

Press the password “321” continuously before the scale begin to do self-check.

Use “” and ”$” to chose

| 1.Pnt | Decimal point | 0； 0.0；0.00 |
| --- | --- | --- |
| 2.TrA | zero tracking range | 0.0; 0.5;1.0; 1.5; 2.0 |
| 3. LIt | Darkness of LED | 0; 1; 2; 3 |